{{applicant_name}}

TEST REPORT

SCOPE OF WORK

PERFORMANCE TESTING ACCORDING TO ECODESIGN REQUIREMENT– LED PRODUCT

LIGHT SOURCE: {{rated_voltage}} {{rated_wattage}} {{cct}}

REPORT NUMBER	
{{report_no}}

ISSUE DATE  		[REVISED DATE] 
October 15, 2025	       [None]

PAGES

17

DOCUMENT CONTROL NUMBER

Test Template _ HZH_ Lamp ERP 2020 _ ED2.0

© 2026  INTERTEK

ECODESIGN REQUIREMENTS FOR LIGHT SOURCES AND SEPARATE CONTROL GEARS OF COMMISSION REGULATION (EU) 2019/2020 & AMENDING REGULATION (EU) 2021/341

Applicant, dentification of the Test Sample

| Applicant | {{applicant_name}} |  |  |
| --- | --- | --- | --- |
| Applicant Address | {{applicant_address}} |  |  |
| Product | Highbay module contained in luminaire |  |  |
| Brand Name | Tospo |  |  |
| Manufacturer | Same as applicant |  |  |
| Light Source Type | {{model_identifier}} |  |  |
| Electrical Rating | 230V AC, 50/60Hz, 100W, 15000 lm, 5000K, Ra≥80, 50000 h, Non-Directional |  |  |
| Mains Efficacy: | 150 lm/w | Energy Efficiency Class* | D |
| Testing Requested | Performance Tests According to Commission Regulation (Eu)2019/2020 |  |  |
| No. of Sample | 10 pcs LED module |  |  |
| Date Received | April 21, 2025 |  |  |
| Date evaluated | April 21, 2025 ~ October 13, 2025 |  |  |

Summary of test results

The test results of light source are evaluated and in compliance with the ecodesign requirements of the Commission Regulation (EU) 2019/2020& AMENDING REGULATION (EU)2021/341.

*Energy efficiency is calculated according to Annex II of Commission DELEGATED REGULATION (EU) No 2019/2015&AMENDING REGULATION (EU)2021/340, and the result is for reference.

Additional Information

## All the information in this table is declared and designated by applicant.

| Test Item: | Containing product light sources separate control gears |  |
| --- | --- | --- |
| For containing product: 230V-100W 5000K | LED module: 230V-100W 5000K | Removable |
|  |  | Replaceable |
|  | Control Gear: N/A | Replaceable |
|  |  | Non-Removable |
| Lighting technology: | LED Fluorescent CFLni OLED |  |
| Intended Use: | Non-directional Directional |  |
| Mains or non-mains: | MLS NMLS |  |
| Connected or not: | YES No |  |
| Colour tuneable: | YES No other |  |
| Tested mode: | N/A |  |
| Rating of Control gear: | N/A |  |

## Standard and Testing Condition

| Issued Laboratory Address Test Location | Intertek Testing Services Zhejiang LTD Hanghou Branch 4th Floor, Building 4#, No. 22, 22nd Street, Qiantang District Hangzhou, China 310018 Hengdian Group Tospo Lighting Co., Ltd. TEST CENTER No. 88 Kexing Road, Hengdian Town, Dongyang City, Jinhua City, Zhejiang Province,China |
| --- | --- |
| Tested at | 230V, 50Hz (±0.2 % during measurement, ±2.0 % during life test) |
| Ambient temp. | 25,0°C±1°C (During measurement except life test) |
| THD | <3,0% (Up to and including the 13th harmonic) |
| Measuring device | Refer to next page equipment list |
| Standard applied (partial used or reference) | EN 60061:1993+all amendments up to A40:2008; EN 60064:1995+A2:2003+A3:2006+A4:2007+A11:2007; EN 60357:2003+A1:2008; EN 60969:1993+A1:1993+A2:2000; CIE 13.3:1995; CIE 15:2004; CIE 18.2:1983; CIE 84:1989; CIE 97:2005; CIE 127:2007; IEC 62722-2-1: 2023; IEC 62722-1: 2014; IEC 62612 ED1.2; IEC 62717-1 ED1.2; IEC 62384 ed.1.1 Commission Regulation EU 2019/2020; Commission Regulation EU 2019/2015; Commission Regulation EU 2021/341; Commission Regulation EU 2021/340; Commission Regulation EU 2023/2048. |

Equipment List

| ## Equipment name | ## Model No | ## Specifications | ## Manufacturer |
| --- | --- | --- | --- |
| Goniophotometers | GO-R5000 | scale A zero position: ≤±0.2° | EVERFINE |
| Integrating sphere | HAAS-2000 | Wavelenth 380nm~780nm; 1000K-100000K; | EVERFINE |
| Digital power meter | PF310 | U:0.1V-600V;I:0.002A-20A;PF:-1.000-+1.000; Hz:10Hz -500KHz; | EVERFINE |
| DC testing power source | DPS1005 | Max.output:1000VA; 40-65Hz; 0-150V:9.2A; 0-300V:4.6A | EVERFINE |
| Integrating Sphere | 2.0m | 2.0m | EVERFINE |
| Light flicker analyzer | LFA-3000 | Sampling Rate ：100kS/s | EVERFINE |

Test and Verification Results

| Article 1 (Clause) | Scope of Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 1.a | Light source |  | N/A |
| 1.b | Separated control gear |  | N/A |
|  | LS and SCG in a containing product | Light source in luminaire | P |

| Annex I (Clause) | Definitions in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| (3) | Directional Light Source: at least 80 % of total luminous flux within a cone with angle of 120° | Non-Directional | N/A |
| (15) | Useful luminous flux Φuse | 15000lm | P |
| - | for NDLS: Φuse = total flux emitted in a solid angle of 4π sr (corresponding to a 360° sphere) | corresponding to a 360° sphere | P |
| - | for DLS with beam angle ≥ 90°: Φuse flux emitted in a solid angle of π sr (a cone with angle of 120°) |  | N/A |
| - | for DLS with beam angle < 90°: Φuse=flux emitted in a solid angle of 0,586π sr (a cone with angle of 90°) |  | N/A |

— End of Page—

Test and Verification Results

| Annex II (Clause) | Energy Efficiency Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 1.(a) | Energy Efficiency Requirements – Light Source |  | P |
|  | Declared on-mode Power Pon (W): | 100W | N/A |
|  | Maximum Allowed Power Ponmax (W): Ponmax = C x (L + Φuse/(F x η)）x R | Ponmax: 136.6W | P |
|  | Threshold efficacy η (lm/W):η for LED: 120.0 |  | P |
|  | End loss factor L (W):1.5for LED L for LED: 1.5 |  | P |
|  | End loss factor L (W):2.0 for CLS |  | N/A |
|  | Efficacy Factor F: 1.00 for NDLS |  | P |
|  | Efficacy Factor F: 0.85 for DLS |  | N/A |
|  | CRI Factor R:0.65 for CRI ≤ 25 |  | N/A |
|  | CRI Factor R:(CRI+80)/160 for CRI > 25 |  | P |
|  | Correction Factor C Depending on Light Source Characteristics in Table 2 |  | P |
|  | NDLS not operating on mains (NMLS): 1.00 |  | N/A |
|  | NDLS operating on mains (MLS): 1.08 |  | P |
|  | DLS not operating on mains (NMLS): 1.15 |  | N/A |
|  | DLS operating on mains (MLS): 1.23 |  | N/A |
|  | Special Light Source Bonus on C |  | N/A |

| Annex II (Clause) | Energy Efficiency Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 1.(a) | Standby power – Light Source |  | N/A |
|  | The standby power Psb of a light source shall not exceed 0.5 W |  | N/A |
|  | The networked standby power Pnet of a connected light source shall not exceed 0.5 W |  | N/A |
|  | The allowable values for Psb and Pnet shall not be added together |  | N/A |

— End of Page—

Test and Verification Results(Cont’d)

| Annex II (Clause) | Energy Efficiency Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 1.(b) | Energy Efficiency Requirements – Separate Control Gear (at full-load) |  | N/A |
|  | Control gear for LED or OLED light sources: |  | N/A |
|  | The no-load power Pno of a separate control gear shall not exceed 0.5 W |  | N/A |
|  | The standby power Psb of a separate control gear shall not exceed 0.5 W |  | N/A |
|  | The networked standby power Pnet of a connected separate control gear shall not exceed 0.5 W |  | N/A |
|  | Psb and Pnet shall not be added together |  | N/A |

| Annex II (Clause) | Functional Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 2. | Functional Requirements – Light Source (Table 4) |  | P |
|  | Colour Rendering Index CRI: 80 ;excpet | refer to test data | P |
|  | For outdoor/Industrial or other <80 application if clear indication on package |  | N/A |
|  | Displacement Factor DF at Power Input Pon for LED and OLED MLS: DF ≥ 0.5 at 5 W < Pon ≤ 10 W, DF ≥ 0.7 at 10 W < Pon ≤ 25 W DF ≥ 0.9 at 25 W < Pon | refer to test data | P |
|  | Lumen Maintenance Factor (for LED and OLED): | Required Min. 96.0% at 3600h | P |
|  | Survival Factor (for LED and OLED): At least 9 light sources of the test sample must be operational after completing the test in Annex V of this Regulation. | refer to test data | P |
|  | Colour consistency for LED and OLED light sources: Variation of chromaticity coordinates within a six-step MacAdam ellipse or less. | refer to test data (Applicant required SDCM<5) | P |
|  | Flicker for LED and OLED MLS: Pst LM ≤ 1.0 at full-load | refer to test data | P |
|  | Stroboscopic effect for LED and OLED MLS: SVM ≤ 0.4 at full-load; except | refer to test data | P |
|  | for outdoor/Industrial or other application allow<80 |  | N/A |

— End of Page—

Test and Verification Results

| Annex II (Clause) | Information Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 3.(a) | Information to be displayed on the light source itself |  | P |
|  | Useful luminous flux (lm) | 15000lm | P |
|  | Correlated colour temperature (K) | 5000K | P |
|  | Beam angle (°) for directional light sources |  | N/A |
| 3.(b) | Information to be visibly displayed on the packaging |  | N/A |
| 3.(b)(1) | Light source placed on the market, not in a containing product |  | N/A |
|  | (a) Useful luminous flux (lm): - In a font at least twice as large as the display of the on-mode power (Pon) - Clearly indicating if it refers to the flux in a sphere (360°), in a wide cone (120°) or in a narrow cone (90°) | Place on the market in a containing product | N/A |
|  | (b) Correlated Colour Temperature, rounded to the nearest 100 K |  | N/A |
|  | (c) Beam angle in degrees for directional light sources |  | N/A |
|  | (d) electrical interface details, e.g. cap- or connector-type, type of power supply (e.g. 230 V AC 50 Hz, 12 V DC) |  | N/A |
|  | (e) L70B50 lifetime for LED and OLED light sources, expressed in hours |  | N/A |
|  | (f) on-mode power (Pon), expressed in W |  | N/A |
|  | (g) standby power (Psb), expressed in W and rounded to the second decimal. If the value is zero, it may be omitted from the packaging |  | N/A |
|  | (h) networked standby power (Pnet) for CLS, expressed in W and rounded to the second decimal. If the value is zero, it may be omitted from the packaging |  | N/A |
|  | (i) Colour Rendering Index, rounded to the nearest integer |  | N/A |
|  | (j) Clear indication to this effect if CRI< 80, and the light source is intended for use in outdoor applications, industrial applications or other applications where lighting standards allow a CRI< 80. |  | N/A |

Test and Verification Results

|  |  |  | (k) Information on non-standard conditions (such as ambient temperature Ta ≠ 25 °C or specific thermal management is necessary) |  |  | N/A |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | (l) a warning if the light source cannot be dimmed or can be dimmed only with specific dimmers or with specific wired or wireless dimming methods. In the latter cases a list of compatible dimmers and/or methods shall be provided on the manufacturer’s website |  |  | N/A |
|  |  |  | (m) if the light source contains mercury: a warning of this, including the mercury content in mg rounded to the first decimal place |  |  | N/A |
|  |  |  | (n) if the light source is within the scope of Directive 2012/19/EU, without prejudice to marking obligations pursuant to Article 14(4) of Directive 2012/19/EU, or contains mercury: a warning that it shall not be disposed of as unsorted municipal waste |  |  | N/A |
|  |  |  | Items (a) to (d) shall be displayed on the packaging in the direction meant to face prospective buyer |  |  | N/A |
|  |  |  | For LS with different characteristics, the reference control setting shall be reported, and |  |  | N/A |
|  |  |  | A range of obtainable values my be indicated |  |  | N/A |
|  |  |  | Above information does not need to exact wording, it may be displayed in graphs, drawings, or symbols |  |  | N/A |
| 3.(b)(2) |  | Separate control gears For separate control gear placed on the market as a stand-alone product, not as a part of a containing product |  |  |  | N/A |
|  | (a) the maximum output power of the control gear (for HL, LED and OLED) or the power of the light source for which the control gear is intended (for FL and HID) |  |  | Place on the market as a part of containing product | N/A |
|  | (b) the type of light source(s) for which it is intended |  |  |  | N/A |
|  | (c) the efficiency in full-load, expressed in percentage |  |  |  | N/A |

— End of Page—

Test and Verification Results

|  | (d) the no-load power (Pno), expressed in W and rounded to the second decimal, or the indication that the gear is not intended to operate in no-load mode. If the value is zero, it may be omitted from the packaging but shall nonetheless be declared in the technical documentation and on websites |  | N/A |
| --- | --- | --- | --- |
|  | (e) the standby power (Psb), expressed in W and rounded to the second decimal. If the value is zero, it may be omitted from the packaging but shall nonetheless be declared in the technical documentation and on websites |  | N/A |
|  | (f) the networked standby power (Pnet), expressed in W and rounded to the second decimal. If the value is zero, it may be omitted from the packaging but shall nonetheless be declared in the technical documentation and on websites |  | N/A |
|  | (g) a warning if the control gear is not suitable for dimming of light sources or can be used only with specific types of dimmable light sources or using specific wired or wireless dimming methods. In the latter cases, detailed information on the conditions in which the control gear can be used for dimming shall be provided on the manufacturer’s or importer’s website |  | N/A |
|  | (h) a QR-code redirecting to a free-access website of the manufacturer, importer or authorised representative, or the internet address for such a website, where full information on the control gear can be found |  | N/A |

— End of Page—

Test and Verification Results

| Annex II (Clause) | Information Requirements in Regulation (EU) 2019/2020 | Result - Remark | Verdict |
| --- | --- | --- | --- |
| 3.(c) | Information to be visibly displayed on a free-access website of the manufacturer, importer or authorised representative |  | N/A |
| 3.(c)(1) | Separate control gears For any separate control gear that is placed on the EU market, the following information shall be displayed on at least one free-access website: |  | N/A |
|  | (a) the information specified in point 3(b)(2), except 3(b)(2)(h) | Input and output rating should be displayed on free-access website | N/A |
|  | (b) the outer dimensions in mm |  | N/A |
|  | (c) the mass in grams of the control gear, without packaging, and without lighting control parts and non-lighting parts, if any and if they can be physically separated from the control gear |  | N/A |
|  | (d) instructions on how to remove lighting control parts and non-lighting parts, if any, or how to switch them off or minimise their power consumption during control-gear testing for market surveillance purposes |  | N/A |
|  | (e) if the control gear can be used with dimmable light sources, a list of minimum characteristics that the light sources should have to be fully compatible with the control gear during dimming, and possibly a list of compatible dimmable light sources |  | N/A |
|  | (f) recommendations on how to dispose of it at the end of its life in line with Directive 2012/19/EU |  | N/A |

— End of Page—

Appendix-Test Data Sheet

1. Initial Photometric and electrical Measurement

{{photometric_data}}

—End of Page—

Appendix-Test Data Sheet

- Energy efficiency classes of light sources under EU 2019/2015

|  | Total mains Efficacy lm/W | Tested EEI (base on average of test result) | EEI Class |  |
| --- | --- | --- | --- | --- |
| Model name | ηΤM = (Φuse / Pon) x FTM |  | Energy efficiency class | ηΤM |
| 230V-100W 5000K | 157.1 | D | A B C D E F G | 210 ≤ ηΤM 185 ≤ ηΤM < 210 160 ≤ ηΤM < 185 135 ≤ ηΤM < 160 110 ≤ ηΤM < 135 85 ≤ ηΤM < 110 ηΤM < 85 |

- Endurance and & Lumen Maintenance Test

Ambient temperature: 25 °C±5°C

| Model | Sample | Switching Test | Luminous | Lumen | Survival |
| --- | --- | --- | --- | --- | --- |
| No. | No. | 1200 cycles | Flux Φuse | Maintenance | Factor |
|  |  | (150min on | (lm) | Factor (%) |  |
|  |  | 30min off) | (3.6 khrs) | (3.6 khrs) | (≥9) |
| 230V-100W 5000K | 1 | Yes | 14955 | 99.38% | Pass |
|  | 2 | Yes | 15025 | 98.87% | Pass |
|  | 3 | Yes | 14996 | 99.34% | Pass |
|  | 4 | Yes | 15025 | 98.85% | Pass |
|  | 5 | Yes | 15041 | 98.63% | Pass |
|  | 6 | Yes | 14827 | 99.38% | Pass |
|  | 7 | Yes | 14964 | 99.42% | Pass |
|  | 8 | Yes | 15151 | 99.56% | Pass |
|  | 9 | Yes | 15197 | 99.42% | Pass |
|  | 10 | Yes | 14879 | 99.10% | Pass |
| Average | -- | Yes | 15006 | 99.19% | 10 |

—End of Page—

Appendix-Test Data Sheet

4. Zonal Lumen Summary and Light Distribution (Representative)

| Zone | Lumens (lm) | % Luminaire (%) |
| --- | --- | --- |
| 0-30 | 4016 | 32.0 |
| 0-60 | 11999 | 79.7 |
| 0-90 | 15048 | 100.0 |
| 0-180 | 15048 | 100.0 |

| Test Item | Results |
| --- | --- |
| Beam Angle | 118.0° |
| Peak Intensity | 5110cd |

— End of Page—

Remark:

- According to declaration of applicant, the product covered by this report is LED luminaire with removable, just the LED module (whole luminaire not included lampshade) base was tested as a light source.

- All photometry and colour test, functional, endurance and lumen maintenance tests were performed at 10 pcs of LED modules.

- “N/A” means test case does not apply to the test object, “TBD” means test result to be determined, “P” means meet the requirement.

- when determining the test conclusion, the Measurement Uncertainty of test has been considered according to Accuracy Method stated in IEC Guide 115.

- The evaluated results tabulated in this report are representative of the actual test samples submitted for this report only.

Attachment:

- Appendix-Product Photos

In Charge of Tests:	Report Reviewed By:

Intertek Testing Services Zhejiang Ltd				   Intertek Testing Services Zhejiang LTD

Hangzhou Branch 				                                 Hangzhou Branch

<Judy Hu>	<Meng Wang >

<Engineer>	<Reviewer>

Lighting Division	Lighting Division

— End of Page—

Appendix -Pictures:

OVERVIEW

Tested light source

{{image_placeholder}}

— End of Report—